# Functional Requirements Specification

## 1. Introduction

## The system requirements were derived directly from the research objectives and sustainability goals of hydroponic farming. These requirements guided architectural decisions, data handling strategies, and the selection of analytics techniques.

## 2. Functional Requirements

### 2.1 Sensor Measurement Requirements

- The system shall measure pH at configurable intervals (default: every 15 minutes).

- The system shall measure EC (electrical conductivity) every 15 minutes.

- The system shall measure water temperature every 15 minutes.

- The system shall measure dissolved oxygen (DO) every 15 minutes.

- The system shall measure ORP (if enabled) every 15 minutes.

- The system shall timestamp each measurement in real time.

- The system shall store sensor calibration metadata.

### 2.2 Data Storage Requirements

- The system shall store all sensor readings locally on ESP32 or SD storage.

- The system shall support optional cloud storage (Firebase/AWS/GCP).

- The system shall buffer data during network outages and resync upon reconnection.

- The system shall maintain at least 30 days of local historical data.

### 2.3 Dashboard & Visualization Requirements

- The system shall provide a web-based or mobile dashboard.

- The dashboard shall display real-time sensor readings.

- The dashboard shall show historical trend charts.

- The dashboard shall allow the user to configure alert thresholds.

### 2.4 Alerting Requirements

- The system shall generate alerts for out-of-range readings (e.g., pH too high).

- Alerts shall be delivered through email, SMS, or push notifications.

- The system shall escalate persistent anomalies (e.g., >10 min).

### 2.5 Analytics & Prediction Requirements

- The system shall apply anomaly detection to sensor data.

- The system shall generate forecasts of pH/EC trends using ML models.

- The system shall compute expected nutrient imbalance risks.

- The system shall provide daily summary analytics.

### 2.6 System Configuration Requirements

- The user shall be able to change sampling rates.

- The user shall be able to calibrate sensors through the UI.

- The system shall store configuration in non-volatile memory.

# Non-Functional Requirements

## 1. Reliability

- System shall operate 24/7 with ≥ 95% uptime.

- Sensor readings shall have <5% allowable data loss.

## 2. Scalability

- Must support additional sensors without major architecture changes.

- Cloud storage must support long-term logging.

## 3. Maintainability

- Codebase shall be modular and easily upgradable.

- Sensor calibration procedures must be documented.

## 4. Security

- Communication between ESP32 and cloud must use TLS/HTTPS.

- Dashboard must require user authentication.

## 5. Energy Efficiency

- ESP32 deep-sleep modes shall be used when continuous monitoring is unnecessary.

- System must support battery backup.

## 6. Environmental Resilience

- Electronics must be protected from humidity (≥90%), water exposure, and nutrient mist.

- Materials must withstand temperatures 5°C–40°C.