# Sensor Overview

## pH Sensors

Operating Principle: Glass electrode measuring hydrogen ion activity.
Accuracy: ±0.1 to ±0.02 pH depending on grade.
Pros: Widely available, accurate, fast response.
Cons: Requires frequent calibration; sensitive to electrical noise.
Cost: €15–€150.
Protocols: Analog (cheap sensors), UART/I²C (smart probes).

## EC (Electrical Conductivity) Sensors

Principle: Measures ionic concentration via conductivity.
Accuracy: ±2–5% of reading.
Pros: Essential for nutrient strength; stable.
Cons: Affected by temperature; calibration needed.
Cost: €25–€180.
Protocols: Analog or UART/I²C.

## Temperature Sensors

Principle: Semiconductor or thermistor sensing.
Accuracy: ±0.5°C typical.
Pros: Cheap, reliable, easy.
Cons: Cable moisture ingress if not sealed.
Cost: €3–€15.
Protocols: 1-Wire (DS18B20), analog, I²C.

## Dissolved Oxygen Sensors

Principle: Galvanic or optical (luminescence).
Accuracy: ±0.2 mg/L (optical) to ±0.5 mg/L (galvanic).
Pros: Critical for root respiration.
Cons: Expensive; membrane replacement.
Cost: €120–€500.
Protocols: Analog or UART.

## ORP Sensors

Principle: Measures oxidation–reduction potential in mV.
Accuracy: ±5 mV typical.
Pros: Good indicator of microbial activity/water cleanliness.
Cons: Slow response; susceptible to drift.
Cost: €30–€200.
Protocols: Analog or UART.