# Data Collection Plan

## 1. Sensor Wiring

- pH/EC/ORP → analog isolation board → ESP32 ADC

- DO sensor → UART

- DS18B20 → 1-Wire pin + resistor

## 2. Sampling Frequency

- pH / EC / Temp: every 30 seconds

- DO / ORP: every 60 seconds

## 3. Calibration Procedures

- pH: 4.0, 7.0, 10.0 buffers

- EC: 1413 μS and 12.88 mS

- ORP: 225 mV and 475 mV standards

- DO: 0% and 100% saturation points

## 4. Data Logging

- ESP32 logs CSV: timestamp, pH, EC, Temp, DO, ORP

- Upload to cloud every 2 minutes

## 5. Safety

- Electrical isolation

- Waterproofing

- Proper handling of calibration chemicals

# Example: Raw Data Log

timestamp,pH,EC,TempC,DO,ORP

2025-01-04 10:00:00,6.1,1.85,22.4,7.1,385

2025-01-04 10:00:30,6.2,1.83,22.4,7.0,382

2025-01-04 10:01:00,6.2,1.84,22.3,7.1,384

Include calibration notes and anomaly markers like:

# anomaly: EC spike detected

# Data Preprocessing Document

### 1. Cleaning Steps

- Remove nulls

- Replace sensor dropouts

- Remove negative or impossible values

### 2. Missing Value Handling

- Linear interpolation

- Forward fill for short gaps

### 3. Normalization

- Min-max scaling for ML

- Standardization (z-score) for anomaly detection

### 4. Filtering

- Rolling average (window 5)

- Median filtering for noise from pumps

### 5. Outlier Detection

- Z-score thresholding

- Isolation Forest

- Statistical control limits (±3σ)

# Exploratory Data Analysis (EDA)

### 1. Time-Series Plots

- pH vs time

- EC vs time

- DO vs time

- Temperature vs time

### 2. Correlations

- pH ↔ EC

- DO ↔ Temperature

- ORP ↔ Nutrient cleanliness

### 3. Trend Analysis

- Daily pH drift

- EC concentration rise

- DO decline with temperature

### 4. Anomalies

- Identify sudden pH drops

- EC spikes

- Sensor noise

### 5. Summary Statistics

- Mean / median / variance

- Min/max

- Rate of change

# Train/Test Split Documentation

### 1. Approach

Time-series split, not random shuffle.

### 2. Ratio

- 70% training (first part of the timeline)

- 30% testing (most recent data)

### 3. Justification

- Time-series must preserve order

- Future forecasting depends on past signals

### 4. Validation Strategy

- Sliding window

- Cross-validation using walk-forward validation